Purchase Specifications for a 
2-3+ Nautical Mile Range Solar Marine Lantern

Overview

This specification is for a 2-3nm+ marine lantern.

Each light shall be entirely self-contained with 1 x 1.4 watt solar panel, 4Ah NiMH battery and ultra-high intensity LEDs.

The lights shall be delivered ready to install. The only assembly required will be activation of each individual light and optional mounting accessories.

1.0 Light Characteristics

The lanterns light source will be one (1) LED.

The lantern output shall be available in red, green, white, yellow, blue.

The lantern shall have a maximum available intensity (dependent on model) of up to:

- Red lantern output 24.7cd

- Green lantern output 26.6cd

- White lantern output 29.2cd

- Yellow lantern output 18.8cd

The lantern shall have a visible range of 2-3nm+.

The lantern shall have a horizontal output of 360 degrees.

The lantern shall have a vertical divergence of 7 degrees.

The lantern shall have up to 256 IALA recommended flash characteristics.

The lantern shall have intensity settings in adjustable in 25% increments.

2.0 Electrical Characteristics

The lantern shall have integrated circuit protection.

The lantern shall have a nominal voltage of 3.6v.

The battery type shall be Nickel Metal Hydride (NiMH).

The battery shall have a capacity of 4Ah.

The lantern shall have an operating temperature range between -40 to 80°C.

3.0 Physical Characteristics

The body of the lantern shall be manufactured from UV stabilised polycarbonate.

The lantern lens shall be UV stabilised polycarbonate or equivalent.

The lantern shall have a mounting pattern using 4 x 4.5mm mounting holes.

The lantern shall have a height of up to 205mm (8⅛ inches).

The lantern shall have a width of 177mm (7 inches).

The lend design shall be single LED optic.

The lantern shall have a mass up to 0.9kg (2 1/8 lbs) dependent on model.

The lantern shall have a life expectancy of up to 12 years.

4.0 Options

The lantern shall be offered with the following options available from the manufacturer:

- 16Ah battery

- 8Ah battery

- 200mm OD base

- 50mm pole mount adapter plate

- IR Programmer

- GPS Synchronisation

5.0 Certifications

The lantern shall be IP68 waterproof.

The manufacturer shall be ISO9001:2008 certified.

The lantern shall meet the following certifications EN61000-6-3:1997.  EN61000-6-1:1997.

The lantern shall meet IALA recommendation E-200-1 Signal colours

6.0 Warranty

The light shall have a three (3) year warranty, excluding battery which will have a warranty of one (1) year.